- O & M MANUAL

Clark Contracts

Babcock Fost Building

GES155

GES ELECTRICAL LTD

42 Victoria Road

Gourock

PA19 1DB

Tel/Fax 01475 713462

##### I N D E X

D E S C R I P T I O N					S E C T I O N  No.

Introduction							1

Warning Notes							2

Operating Instruction						3

Servicing								4

Light Fittings/Spares and Parts List			5

Distribution Panel/Parts List				6

Cable/Miscellaneous List					7

Inspection & Test Certificates				8

Contract Drawings						9

S E C T I O N   O N E.

## Introduction.

GES ELECTRICAL LTD carried out the electrical installation at Babcock Fost Building.

This operation manual provides detailed information for maintenance engineers in the future for guidelines of replacement part and guidelines to maintenance.

No arbitrarily ruling can be given regarding the length of time to lapse between periodic inspection as obviously this depends on the client and the infrastructure of maintenance, security, site environmental and climate conditions. We as a company would recommend that the client implements a maintenance schedule to be carried out on a regular basis, experienced personnel in the electrical field should implement this and a form of record left in this file.

It should be noted that before carrying out any maintenance checks or adjustments of any electrical components, the equipment should be isolated and locked off from the source of electrical power supply.

For and on behalf of

GES ELECTRICAL LTD

S E C T I O N   T W O.

WARNING NOTES.

- Do not fit fuses and MCB of larger capacity than supplied within the electrical distribution panel or any fusible item.

- Make sure a full compliment of spare parts e.g. lamps for light fittings are always available, particularly parts most often replaced, and a record left.

- Maintenance should be carried out by experienced, qualified electrical personnel only and safe isolation procedures implemented at all times when works are being carried out.

- The cause of any circuit breaker to trip must be investigated and eliminated at all times

before the power be reapplied to any circuit.

S E C T I O N   T H R E E.

### Operating Instructions.

Electrical Distribution

- The main electrical distribution is situated in the basement level of the building. Scottish power cable head is situated to the left hand side of the main electrical panel’ with there 3-phase 4.5v main switch situated on the wall next to the mains panel, this switch if switched to the “OFF” position would isolate the main incoming power supply to the electrical installation within the building.

- Localised isolation of circuits can be done from the main electrical panel. All circuit breakers have a legend explaining a brief detail of its operation located to the centre of the panel. Each circuit breaker clearly states whether it is in the “ON” position or the “OFF” position for maintenance purposes circuit breakers should be locked off and labelled clearly that personnel are working on circuit, we recommend that a clear procedure be put in place by client to ensure that this is adhered to at all times and by Experienced electrical personnel only.

Lighting

- All light fittings are operated via localised switches in each room of building.

Emergency Lighting

- Next to each light switch an emergency light test switch is located this is for maintenance purposes only – please refer to emergency lighting section for a more detailed account of operation.

Power Socket Outlets

- All socket outlets are 240v 13amp 3-pin outlets, please note that all appliances should have correct fuse value installed and be certified portable appliance tested. All socket outlets have RCD protection.

Fire Alarm

- Fire Alarm System is a fully automated system, consisting of fire alarm panel situated at front entrance, combined sounder and smoke detection situated at the roof level and emergency break glasses situated at each exit/entrance to building.

Outside Lighting

1.   Outside lighting can be operated via switch in the main reception area located on side wall of I.T. cupboard.

S E C T I O N   F O U R.

Servicing.

SERVICING OF THE CONTROL GEAR, SWITCH GEAR, LIGHTING, FIRE ALARM SYSTEM – Should be carried out by Experience Personnel only.

- Inspect for loose connections, and tighten nuts, bolts, electrical connections, care should be taken when removing any covers, always make sure all exposed electrical connections have been isolated before carrying out any servicing.

- Examine all switches, control gear, distribution, and lighting socket outlets for signs of overheating or wear and tear, replace if necessary.

- Check all fuses have the correct rating and are in healthy condition.

- Log and report all servicing and list all new parts within this folder.

S E C T I O N   F I V E.

Light Fittings/Lamps

600X600 DIMABLE FLAT PANEL LED

600X600 DIMABLE FLAT PANEL EM TYPE LED

600X600 NON DIMABLE SURFACE MOUNTED FLAT PANEL LED

24W CAT 6 PATCH PANEL

S E C T I O N    S I X.

Distribution Panels/Parts List

S E C T I O N    S E V E N.

Cable/Miscellaneous

FP200 1.5MM 2C+E WHITE REF: FP200GOLD1

6MM GREEN/YELLOW EARTH WIRE REF: 6491B6G

6MM X 6MM YELLOW CRIMPS REF: QYR6

32A ROTARY ISOLATOR

CABLE 2.5MM 3C XLPA/SWA/PVC 100M REF: 6943XLH2.5BL:GK

G/BROOK 0.5” CABLE CLEAT BLACK REF: NP5:GK

EUROPA LB204P ROTARY ISOLATOR 4P 20A REF: NP5:GK

779332 CAT 6 EURO MODULE REF: NS1: GK

CAT 6 FACEPLATE 2 MOD 1GANG SIZE

6 WAY EARTH BAR

32A 3PIN COMANDO SOCKET

1 GANG 1WAY SWITCH

1 GANG 25MM DEEP SPFT PATTRESS BOX

M TUFFEX SNG INSERTS FOR DADO

20MM CONDUIT PVC

MODE 10A PLATESWITCH 1G 2W REF: CMA0110

PATTRESS PVC 1G 32MM ROUND WHITE REF: CMP8132

MT CR2WH CONDUIT LG 20MMX3M WHITE REF: CR2WH

MT MSB2WH SPACER BAR SADDLE 20MM WHI REF: MSB2WH

INT/SKT 32A 240V 2P+E IP44 GEWISS REF: GW66015

MT ESSB1WH BOX 1 GANG 30MM WHI REF: ESSB1WHI

S  E C T I O N   E I G H T.

Inspection & Test Certificates

S E C T I O N   N I N E.

Contract Drawings